Orbit Odyssey — Computer Vision
Complete Competition Manual

David Alejandro Fuquen
2026-06-01

# Introduction & Mission

Welcome to the Orbit Odyssey Computer Vision Challenge! Two teams compete simultaneously on a shared arena, each training an object detection model, deploying it on an Intel AIxBoard, and letting an XRP robot act autonomously based on what it sees through its camera. You will not control the robot during the challenge: your model and your code determine how well the robot performs.

This manual covers everything in one place: the arena layout, object behaviors, competition rules, scoring, model training, competition day procedures, moderator instructions, and a printable scorecard.

## The Mission

The year is 2147. A cargo shuttle carrying critical supplies has crash-landed on the surface of a remote moon. The impact scattered debris across the crash site, and several escape pods were ejected on impact and now lie among the wreckage. Two rival engineering teams have each been tasked with deploying an autonomous rescue rover to opposite halves of the crash site.

Your rover must navigate the debris field:

- Contact each escape pod (Rocket): approach it, make physical contact to activate its homing beacon, then spin clockwise and continue the search.

- Avoid hazardous debris: some wreckage is unstable.

  - Tire debris must be avoided by spinning clockwise.

  - Tin Can debris must be avoided by performing an evasive maneuver: a brief counter-clockwise feint (20°) followed by a clockwise sweep (60°).

- Touching any debris triggers a containment breach.

The crash site is divided by a partition wall. Each team operates on one half. There is a catch: the rover’s communication window lasts only 120 seconds. After that, the signal is lost and the mission ends. Your team’s only decision is which direction the rover faces at deployment. From that moment on, the rover must rely entirely on its camera and your trained model to identify objects and decide what to do.

The better your model, and the smarter your code, the better the rover performs.

# The Arena

## Field dimensions

The playing field is a rectangular arena enclosed by low-profile PVC pipe barriers on all four sides, with a central partition dividing it into two halves. This is the exact same field used in the original Orbit Odyssey challenge.

Field dimensions and boundaries.

| Property | Value |
| --- | --- |
| Total length (X-axis) | 254.00 cm (100 in) |
| Total width (Y-axis) | 142.24 cm (56 in) |
| Total floor area | 3.61 m2 |
| Perimeter walls | Low-profile PVC pipe barriers |
| Corner zones | 30.48 × 30.48 cm (12 × 12 in) at each corner |

The coordinate system places the origin (0,0) at the bottom-left corner of the field. The X-axis runs left to right (0 to 254 cm) and the Y-axis runs bottom to top (0 to 142.24 cm). The partition does not block the camera’s line of sight (the camera sits above pipe height). It blocks the robot’s driving path through the center 50 inches, confining each team’s robot to its own half.

## Mirrored layout

Both teams use the same triangle arrangement, mirrored across the partition. Team A plays in the bottom half; Team B plays in the top half. Each half contains 3 objects and one robot starting position.

Mirrored P_3 layout — each team has 3 objects in its half.

## Object positions

Object positions for both halves.

| ID | Location | X (in) | Y (in) | X (cm) | Y (cm) | Half |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | Lower-left | 20 | 8 | 50.80 | 20.32 | Bottom |
| P2 | Lower-right | 80 | 8 | 203.20 | 20.32 | Bottom |
| P3 | Above center | 50 | 20 | 127.00 | 50.80 | Bottom |
| P1’ | Upper-left | 20 | 48 | 50.80 | 121.92 | Top |
| P2’ | Upper-right | 80 | 48 | 203.20 | 121.92 | Top |
| P3’ | Below center | 50 | 36 | 127.00 | 91.44 | Top |

Robot starting positions.

| Robot | Location | Position (cm) | Position (in) |
| --- | --- | --- | --- |
| Robot A | Bottom half, center of triangle | (127.00, 30.48) | (50, 12) |
| Robot B | Top half, center of triangle | (127.00, 111.76) | (50, 44) |

## Distance from robot to each object

Distance from robot center to each object (identical for both halves due to symmetry).

| ID | Distance to robot (cm) | Distance to robot (in) |
| --- | --- | --- |
| P1 (or P1’) | 76.9 | 30.3 |
| P2 (or P2’) | 76.9 | 30.3 |
| P3 (or P3’) | 20.3 | 8.0 |

## Clearance verification

Minimum clearances: object center to wall ≥ 20 cm, object center to partition ≥ 20 cm, object center to object center ≥ 35 cm, objects must be outside all corner zones.

Clearance verification (all values in cm). Identical for both halves by symmetry.

| ID | Left wall | Right wall | Outer wall | Partition | Min wall | Nearest obj. |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | 50.8 | 203.2 | 20.3 | 50.8 | 20.3 | 82.1 (P3) |
| P2 | 203.2 | 50.8 | 20.3 | 50.8 | 20.3 | 82.1 (P3) |
| P3 | 127.0 | 127.0 | 50.8 | 20.3 | 20.3 | 82.1 (P1) |

## Class rotation templates

Object positions are physically fixed. Between rounds, the referee assigns classes to positions using one of these templates. Both halves use the same template simultaneously.

Class rotation templates. Each template ensures all 3 classes are present.

| Template | P1 / P1’ | P2 / P2’ | P3 / P3’ |
| --- | --- | --- | --- |
| A | Rocket | Tire | Tin Can |
| B | Tin Can | Rocket | Tire |
| C | Tire | Tin Can | Rocket |

## Navigation notes

- Each team’s robot operates entirely within its own half. The partition physically prevents crossing.

- The robot starts at the centroid of the triangle. P3 (or P3’) is closest (∼20 cm), directly above (or below). P1 and P2 are ∼77 cm away, near the left and right walls.

- The robot can reach any object by spinning to face it and driving forward.

# The Objects & Robot Behaviors

There are three object classes. Each is a 3D-printed object with an approximate radius of 6 cm.

## Object behaviors and scoring

Object classes and correct robot actions.

| Object | Type | Correct Robot Action | Points |
| --- | --- | --- | --- |
| Rocket | Contact | Approach, touch object, spin clockwise | +20 |
| Rocket | Contact (no touch) | Approach, spin clockwise (without touching) | +15 |
| Tire | Avoid | Approach, spin clockwise, drive away (no touch) | +15 |
| Tin Can | Avoid | Approach, spin CCW 20° then CW 60°, drive away (no touch) | +15 |

### Tin Can compound rotation

The Tin Can avoidance requires a two-part rotation:

- Robot approaches the Tin Can to ∼20 cm.

- Robot spins counter-clockwise 20° (a brief feint).

- Robot then spins clockwise 60° (the main evasion).

- Robot drives away without touching the object.

The net result is approximately 40° clockwise. The referee must observe both rotation phases for the engagement to score as a correct avoid (+15). If the robot performs only a clockwise spin (missing the initial counter-clockwise feint) or any other pattern, it is scored as a wrong-direction avoid (+5).

The Approach Requirement

For any engagement to count—contact, approach, or avoid—the robot must visibly drive toward the object and reach approximately 20 cm or closer before executing any spin or contact action.

Random spinning does not count. If the robot rotates in place near an object without first making a clear, deliberate approach, the referee will not score that engagement. The referee must observe a distinct forward movement toward the target object before any rotational action is performed.

This rule prevents uncontrolled spinning from being mistaken for intentional behavior. The robot must demonstrate that it recognized the object and chose to interact with it.

## First recognition only

Each object is scored exactly once: the first time the robot engages it. Maximum 3 scored engagements per run (one per object). Subsequent interactions with the same object are ignored for scoring purposes.

## Misclassification consequences

If the model misclassifies an object (for example, labels a Tire as a Tin Can) the robot will perform the wrong rotation. This earns partial credit (+5, wrong-direction avoid) but costs the full +15.

If the robot physically touches an avoid object, that is a false contact (−10) and also disqualifies the team from the Clean Avoid bonus (+15). A single false contact represents a net swing of −25 points compared to a correct avoid.

# Rules of the Game

- Game duration. Each run lasts exactly 120 seconds. Two teams play simultaneously, one in each half of the field. The timer starts on the referee’s signal and never pauses. When the buzzer sounds, the run is over.

- Starting position. Each robot is placed at its designated starting position: Team A at (127.00, 30.48) cm in the bottom half, Team B at (127.00, 111.76) cm in the top half. The robot starts at the centroid of its object triangle.

- Initial heading. The team selects the direction the camera faces at the start. This is the only human decision permitted during the challenge.

- Hands-off rule. From the start signal to the buzzer, no team member may touch the robot, any object, the field surface, or the field walls for any reason. There are no exceptions. Any unauthorized touch immediately terminates the run for that team, with the score recorded up to that point.

- Code modification. Teams may modify both main.py (robot behavior, MicroPython) and run_model.py (inference script) to customize robot behavior and inference parameters. However, the trained ONNX model file must still be produced through the provided gui_train_model.py pipeline, which generates the cryptographic receipt required for scoring.

- Autonomy requirement. The robot must operate fully autonomously. The following are prohibited:

  - Remote control of any kind (Bluetooth, Wi-Fi, IR, etc.).

  - External signals (verbal commands, gestures, lasers, lights).

  - Offboard computation (no cloud calls or external processing).

  - Pre-programmed waypoints or hardcoded coordinates.

- Hardware modifications. Only the camera height and tilt angle may be adjusted. No other hardware modifications are permitted: no added sensors, bumpers, changed wheels, mechanical arms, or chassis alterations.

- Object placement. The referee places objects at the specified coordinates using the selected rotation template. Both halves use the same template (mirrored). Teams may not move or interfere with objects.

- Class rotation. Object positions are physically fixed. Between rounds, the referee changes the rotation template, which determines which class is assigned to each position. All teams in the same round face the same assignment.

- No rescue. There is no rescue procedure. If the robot becomes stuck, tips over, or is otherwise unable to continue, the run continues until the 120-second timer expires. No team member may enter the field or touch the robot for any reason during the run.

- Partition rules. The partition is a physical barrier separating the two halves. Colliding with the partition or perimeter walls is not a penalty. Each robot must remain in its designated half. If a robot navigates through a gap into the other half, the round will be terminated and the robot will be taken out of the field so the other team is not interrupted.

- Sportsmanship. Teams must conduct themselves respectfully. Deliberate interference with the opposing team’s robot, objects, or field half is grounds for disqualification.

# Scoring System

## Total score formula

Total Score = (Field Performance × Model-Size Multiplier) + mAP Bonus + Performance Bonus

Total=(A×B)+C+D

## A — Field Performance

The sum of points earned from object engagements during the 120-second run. Each object is scored at most once (first engagement only). Maximum 3 scored engagements per run.

Scoring actions and point values.

| Action | Points | Referee observes |
| --- | --- | --- |
| Correct Rocket contact | +20 | Approach, touch, CW spin |
| Correct Rocket approach (no touch) | +15 | Approach, CW spin, no touch |
| Correct Tire avoid | +15 | Approach, CW spin, no touch |
| Correct Tin Can avoid | +15 | Approach, CCW 20° then CW 60°, no touch |
| Wrong-direction avoid | +5 | Approached but wrong rotation pattern |
| False contact (touch avoid object) | −10 | Avoid object visibly displaced or touched |
| No interaction | 0 | Object not engaged |

Maximum Field Performance = 20+15+15=50 points (all 3 objects engaged correctly, Rocket touched).

There is no time bonus. If the robot finishes interacting with all objects before the 120-second timer expires, the round continues until the buzzer.

## B — Model-Size Multiplier

Determined from the model variant recorded on the cryptographic receipt.

Model-size multiplier tiers.

| Tier | Variant | Multiplier |
| --- | --- | --- |
| Small | nano, small | × 1.05 |
| Medium | medium | × 1.00 |
| Large | large, xlarge | × 0.95 |

## C — mAP Bonus (max +15)

Computed before the run from the per-class mAP50 values on the cryptographic receipt:

mAP Bonus=avg(Rocket_mAP, Tire_mAP, Can_mAP)×15

Example: mAP values of 0.92, 0.87, 0.81 → avg = 0.867 → bonus = 0.867×15=13.0.

## D — Performance Bonus (max +30)

Performance bonuses. These are not multiplied by the model-size multiplier.

| Bonus | Condition | Points |
| --- | --- | --- |
| Perfect Contact | The Rocket was approached and physically touched | +15 |
| Clean Avoid | No avoid object was touched during the entire 120s | +15 |

## Maximum possible score

Maximum score breakdown.

| Component | Max value | Condition |
| --- | --- | --- |
| A (Field Performance) | 50 | All 3 objects correct, Rocket touched |
| B (Multiplier) | × 1.05 | nano/small model |
| A×B | 52.50 |  |
| C (mAP Bonus) | +15.00 | Perfect mAP across all classes |
| D (Performance Bonus) | +30.00 | Perfect Contact + Clean Avoid |
| Maximum Total | 97.50 |  |

## Worked examples

### Example 1: Strong performance

nano model (×1.05), avg mAP = 0.90. Robot contacts Rocket (+20), correctly avoids Tire with CW spin (+15), correctly avoids Tin Can with CCW 20° + CW 60° (+15). No avoid objects touched.

A&=20+15+15=50D&=15 (Perfect Contact)+15 (Clean Avoid)=30Total&=(50×1.05)+(0.90×15)+30=52.50+13.50+30=96.00

### Example 2: Mixed performance

medium model (×1.00), avg mAP = 0.78. Robot approaches Rocket but doesn’t touch (+15). Tire: robot spins wrong direction (+5). Tin Can: correct compound avoid (+15). No avoid objects touched.

A&=15+5+15=35D&=0 (Rocket not touched)+15 (Clean Avoid)=15Total&=(35×1.00)+(0.78×15)+15=35.00+11.70+15=61.70

### Example 3: Poor performance

large model (×0.95), avg mAP = 0.65. Robot contacts Rocket (+20). Robot touches Tire during approach, false contact (−10). Tin Can not engaged (0).

A&=20+(−10)+0=10D&=15 (Perfect Contact)+0 (Tire touched → no Clean Avoid)=15Total&=(10×0.95)+(0.65×15)+15=9.50+9.75+15=34.25

# Cryptographic Receipt

When you export your model to ONNX, the training pipeline automatically generates a cryptographic receipt. This receipt is your team’s proof of model integrity.

## What it contains

Cryptographic receipt fields.

| Field | Description |
| --- | --- |
| Model file hash | SHA-256 hash of the exported ONNX file |
| Model variant | Architecture used (e.g., nano, small, large) |
| Per-class mAP50 | mAP value for each of the 3 classes |
| Export timestamp | Date and time the model was exported |

## Why it matters

Before your run, the referee will:

- Compute the SHA-256 hash of the ONNX file on your AIxBoard.

- Compare it to the hash on your receipt—they must match exactly.

- Read your model variant to determine the model-size multiplier.

- Read your mAP values to compute the mAP Bonus.

## What if you don’t have one?

If you cannot present a valid receipt, or the hash does not match:

- Your mAP Bonus is set to 0.

- Your Model-Size Multiplier is set to 0.95 (Large tier assumed).

- You may still compete and earn Field Performance points, but you lose all pre-run scoring advantages.

Do not lose your receipt. Keep a digital copy. Do not modify the ONNX file after export: any change will invalidate the hash.

# Training Your Model

## Challenge pipeline

The challenge follows a four-stage pipeline:

- Data gathering
Each team takes photos of the 3 object classes for training. A minimum of 200 photos per object is recommended, from as many angles as possible.

- Model training
A GUI application lets you:

  - Select and merge datasets for 3 object classes.

  - Choose a model variant (nano, small, medium, large, xlarge).

  - Train the model and view precision/recall and loss charts.

  - Export the trained model to ONNX format, which generates the cryptographic receipt.

- Inference on AIxBoard
The run_model.py script runs your trained model on a live webcam feed and sends detection results to the XRP robot over serial (UART). Teams may modify this script to customize inference behavior.

- Robot behavior
The main.py script on the XRP robot receives detections and executes the search/contact/avoid state machine. Teams may modify this script to customize robot behavior.

Key takeaway: Teams differentiate themselves through their trained model and their code modifications to main.py and run_model.py. A model that accurately detects and classifies objects—combined with well-tuned robot behavior—produces the best performance.

## Step 1: Prepare datasets

Take images of all 3 classes (Rocket, Tire, Tin Can). Vary the angle, distance, lighting, and background in your photos.

## Step 2: Launch the training GUI

- Select datasets. Browse to each class’s dataset folder. The GUI merges them and remaps class IDs automatically.

- Choose a model variant. See the Strategy Tips section for tradeoffs.

- Set training parameters. The GUI provides default values. You may adjust epochs and batch size.

- Start training. Monitor precision/recall and loss charts in real time.

- Export to ONNX. This generates the ONNX model file and the cryptographic receipt.

## Step 3: Evaluate your model

After training, review the metrics:

- Per-class mAP50 — how well your model detects each class. Directly determines your mAP Bonus.

- Precision and Recall — precision measures false positives; recall measures false negatives.

- Loss curves — should decrease over training. Plateau may indicate need for more epochs or data.

Tip: Train multiple models with different variants and compare mAP scores. The only model that matters is the one you deploy. Trial and error is fundamental to learning!

# Competition Day

## Before your run

- Check in. Present your cryptographic receipt to the referee.

- Verification. The referee computes the SHA-256 hash of your ONNX model on the AIxBoard, verifies it against your receipt, and records your model tier and mAP Bonus on the scorecard.

- Briefing. The referee announces which half each team will play on (Bottom or Top).

## Placing the robot

- Place the robot at the designated starting position in your assigned half.

- Choose the robot’s initial heading (the direction the camera faces). This is your only decision.

- Step back. Once placed, you may not touch the robot again.

## The 120-second run

- The referee gives the start signal: “Ready…Set…Go!”

- The 120-second timer starts. Both robots operate simultaneously.

- You watch. You may not touch the robot, any object, the field, or communicate with the robot in any way.

- If the robot gets stuck, there is no rescue. The run continues until the buzzer.

- The buzzer sounds. The run is over. Any action not completed before the buzzer does not count.

## After the run

The referee tallies the score, applies bonuses, and announces the total. You will see the completed scorecard.

## Strategy tips

### Model size: the core tradeoff

| Choice | Tradeoff |
| --- | --- |
| nano / small | Faster inference, ×1.05 multiplier bonus. May have lower accuracy if dataset is small. Best if you can achieve high mAP. |
| medium | Balanced. No bonus or penalty (×1.00). A safe default. |
| large / xlarge | Highest potential accuracy, but ×0.95 penalty. Only worth it if accuracy gain significantly outweighs the 5% penalty. |

The math: A nano model with 50 Field Performance scores 50×1.05=52.50. A large model needs 52.50/0.95=55.3 Field Performance to match—but the maximum is 50. With 3 objects and first-recognition-only, the small model bonus is especially valuable.

### mAP matters twice

- Directly: the mAP Bonus adds up to +15 points.

- Indirectly: higher mAP means more accurate classification during the run, leading to correct rotations (+15/+20) instead of wrong-direction avoids (+5) or false contacts (−10).

### Heading selection

Your only decision is which direction the camera faces at deployment. The robot starts at the centroid of the triangle, surrounded by the 3 objects:

- P3 is directly ahead (above/below) and closest (∼20 cm). Pointing toward it finds it immediately.

- P1 and P2 are to the lower-left and lower-right (∼77 cm away), near the side walls.

- The robot is in the middle of all 3 objects—any heading will find an object quickly.

### The Clean Avoid bonus

The +15 Clean Avoid bonus is lost if the robot touches any avoid object even once. A single false contact costs −10 and the +15 bonus—a net swing of −25 points. Tuning your code to maintain safe distance while avoiding is critical.

### Code customization

Since teams may modify main.py and run_model.py, consider:

- Tuning approach distances and spin angles to match the required behaviors exactly.

- Implementing the Tin Can compound rotation (CCW 20° then CW 60°) precisely.

- Adjusting search patterns to find all 3 objects efficiently within 120 seconds.

## Frequently asked questions

- Can we modify main.py or run_model.py?
Yes. Teams may modify both scripts to customize robot behavior and inference parameters.

- Can we use a model not trained with gui_train_model.py?
No. The model must be produced through the provided training pipeline, which generates the cryptographic receipt.

- Can the robot score the same object more than once?
No. Each object is scored exactly once—the first engagement counts. Maximum 3 scored engagements per run.

- What if the robot turns the wrong direction on an avoid object?
You get +5 (partial credit). But you lose the difference compared to a correct avoid (+15).

- What counts as a “correct” Tin Can avoid?
The robot must perform the full compound rotation: counter-clockwise 20° followed by clockwise 60°. A simple clockwise-only spin scores as wrong-direction (+5).

- What if the robot bumps into a wall or the partition?
Not a penalty. Collisions with walls and the partition are expected.

- What if the serial connection drops mid-run?
Not a penalty. The game restarts.

- What if the robot gets stuck?
There is no rescue. The run continues until the timer expires. Plan your code to minimize getting stuck.

- Can we adjust the camera between runs?
Yes. Camera height and tilt angle may be adjusted between runs (not during).

- What happens if we lose our cryptographic receipt?
Your mAP Bonus becomes 0 and you are assigned the Large tier multiplier (×0.95). You can still compete but lose significant scoring advantages.

- Do both teams in a round face the same field?
Yes. Both teams use the same rotation template. The layout is mirrored across the partition so each half is identical.

# Moderator Guide

This section provides step-by-step instructions for referees running the competition.

## Terminology

| Term | Definition |
| --- | --- |
| Round | A run in which two teams compete simultaneously under the same field configuration |
| Rotation template | The mapping of object classes to positions (A, B, or C) |
| Mirrored layout | Both halves use the same triangle arrangement, reflected across the partition |
| Engagement | One robot-to-object interaction (approach + action) |
| False contact | Robot physically touches an avoid object |
| Approach | Robot visibly drives toward an object and reaches ≤20 cm |

## Pre-event preparation

Equipment checklist:

- Field: 254 × 142.24 cm with PVC walls

- Partition: 127 cm PVC barrier at Y = 71.12 cm, X = 63.5 to 190.5 cm

- Objects: 2 Rockets + 2 Tires + 2 Tin Cans (one set per half)

- Printed scorecards (2 per run—one per team)

- 120-second timer with buzzer

- Rotation template list (A, B, C)

- SHA-256 hash verification tool

- Pens, measuring tape

Staffing: Two referees are recommended—one per half—so each can observe their team’s robot independently.

## Field setup checklist

- Verify field dimensions (254 × 142.24 cm).

- Install partition at Y = 71.12, X = 63.5 to 190.5 cm.

- Select rotation template for the round (A, B, or C).

- Place objects in both halves using the coordinates below:

Object placement coordinates for both halves.

| Bottom Half (Team A) |  |  |  | Top Half (Team B) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Location | X (cm) | Y (cm) | ID | Location | X (cm) | Y (cm) |
| P1 | Lower-left | 50.80 | 20.32 | P1’ | Upper-left | 50.80 | 121.92 |
| P2 | Lower-right | 203.20 | 20.32 | P2’ | Upper-right | 203.20 | 121.92 |
| P3 | Above center | 127.00 | 50.80 | P3’ | Below center | 127.00 | 91.44 |

- Spot-check clearances: object center to wall ≥ 20 cm, object center to object center ≥ 35 cm, object center to partition ≥ 20 cm.

- Mark robot starting positions: (127.00, 30.48) cm and (127.00, 111.76) cm.

## Verification procedure (per team, before run)

- Request the team’s cryptographic receipt.

- Compute the SHA-256 hash of the ONNX file on the AIxBoard.

- Verify the hash matches the receipt exactly.

- Determine model tier: nano/small → Small (×1.05), medium → Medium (×1.00), large/xlarge → Large (×0.95).

- Compute mAP Bonus: avg(Rocket,Tire,Can)×15.

- Record tier, multiplier, and mAP Bonus on the scorecard.

- If receipt is missing or invalid: mAP Bonus = 0, Multiplier = 0.95.

Repeat for both teams.

## Pre-run procedure

- Verify receipt verification is complete for both teams.

- Fill scorecard headers (team name, date, round #, half assignment).

- Assign halves: Team A → Bottom, Team B → Top.

- Each team places their robot at the designated starting position and selects heading.

- Verify both robots are at correct positions.

- Verify all hands off.

- Give start signal: “Ready…Set…Go!” Start 120-second timer.

## During the run — observation guide

Each referee observes the robot in their assigned half. For each engagement, record:

- Position ID (P1, P2, or P3)

- Object class (from rotation template)

- Action observed

- Whether it was correct (Y/N)

- Points awarded

Maximum 3 scored engagements per team (first recognition only).

### Judging the approach requirement

Before scoring any engagement, verify that the robot visibly drove toward the object. The referee must observe:

- A clear forward movement in the direction of the object.

- The robot reaching approximately 20 cm or closer to the object.

- Only then does the robot execute a spin or contact action.

If the robot spins near an object without a clear approach, the engagement is not scored. This prevents random or uncontrolled spinning from being credited.

### Judging each action type

- Rocket Contact (+20): Robot approaches, physically touches (object moves visibly), then spins clockwise. Both contact and CW spin must be observed.

- Rocket Approach (+15): Robot approaches to ∼20 cm, spins clockwise, but does not touch. CW spin must be observed.

- Tire Correct Avoid (+15): Robot approaches to ∼20 cm, spins clockwise, drives away without touching.

- Tin Can Correct Avoid (+15): Robot approaches to ∼20 cm, performs the compound rotation: counter-clockwise ∼20° then clockwise ∼60°, drives away without touching. The referee must observe both rotation phases.

- Wrong-direction Avoid (+5): Robot approached an avoid object but performed the wrong rotation (e.g., only CW on a Tin Can, or CCW on a Tire).

- False Contact (−10): Robot physically touches an avoid object. Any visible contact counts, even without displacement.

## Post-run procedure

- Call “Time!” when the buzzer sounds.

- Tally Field Performance (A): sum of all engagement points.

- Check Performance Bonuses (D):

  - Perfect Contact: Was the Rocket approached and touched? → +15.

  - Clean Avoid: Was no avoid object touched during the entire run? → +15.

- Calculate final score: Total=(A×B)+C+D.

- Announce score to the team.

- Sign scorecard.

- Reset field: return all objects to their positions for the next run.

## Between rounds

- Select a new rotation template (A, B, or C).

- Swap object classes between positions (positions are fixed; classes change).

- Verify all positions are correct before the next round.

## Quick reference card

| Action | Points |
| --- | --- |
| Correct Rocket contact | +20 |
| Correct Rocket approach (no touch) | +15 |
| Correct Tire avoid (CW spin) | +15 |
| Correct Tin Can avoid (CCW 20° + CW 60°) | +15 |
| Wrong-direction avoid | +5 |
| False contact (touch avoid obj.) | −10 |
| Perfect Contact bonus | +15 |
| Clean Avoid bonus | +15 |
| Small model (nano/small) | × 1.05 |
| Medium model | × 1.00 |
| Large model (large/xlarge) | × 0.95 |
| mAP Bonus | avg_mAP × 15 |

Formula: Total = (A×B)+C+D

## Edge-case quick reference

Edge-case rulings.

| Situation | Ruling |
| --- | --- |
| Robot knocks object off field | Score the engagement normally; object is not replaced. |
| Object moves without robot contact | Does not count as an engagement. If displaced >5 cm, referee may reposition between runs. |
| Robot collides with partition or wall | Not a penalty. |
| Action incomplete when buzzer sounds | Does not count. Contact: object must be displaced ≥3 cm. Avoid: spin must be completed. |
| Serial connection drops mid-run | Not a penalty; game restarts. |
| Object placed in wrong position | During run: stop and replay. After run: score stands. |
| Robot tips over | Run continues. No intervention permitted. |
| Robot drives over avoid object | Any contact = false contact (−10), even without visible displacement. |
| Rocket contact without CW spin | Does not score as contact. Must observe approach + touch + CW spin. |
| Team member enters field during run | Run is immediately terminated for that team. |
| Robot spins without approaching | Does not score. Referee must observe clear approach before spin. |
| Wrong compound rotation on Tin Can | Scored as wrong-direction avoid (+5). E.g., only CW without initial CCW. |
| Robot crosses into other team’s half | Not a penalty; wastes the team’s time. If it interferes with the other robot, referee notes it. |

# Printable Scorecard

Orbit Odyssey — Referee Scorecard
Computer Vision Challenge

| Team Name |  | Date |  | Round # |  |
| --- | --- | --- | --- | --- | --- |
| Layout | P_3 (Mirrored) | Template | A / B / C |  |  |
| Half | ▫ Bottom (A) ▫ Top (B) |  |  |  |  |

Pre-Run Verification (from cryptographic receipt — fill before the run starts)

| Model |  | Tier | ▫ Small (×1.05) ▫ Medium (×1.00) ▫ Large (×0.95) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mAP Rocket |  | mAP Tire |  | mAP Can |  | Avg mAP |  |
| mAP Bonus = Avg × 15 = |  | Multiplier = |  |  |  |  |  |

Field Performance (one row per object — first recognition only, max 3 engagements)

| # | Position | Object Class | Action Observed | OK? | Pts |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | ▫ Contact ▫ Approach ▫ Avoid CW ▫ Avoid CCW→CW ▫ None | ▫Y ▫N |  |
| 2 |  |  | ▫ Contact ▫ Approach ▫ Avoid CW ▫ Avoid CCW→CW ▫ None | ▫Y ▫N |  |
| 3 |  |  | ▫ Contact ▫ Approach ▫ Avoid CW ▫ Avoid CCW→CW ▫ None | ▫Y ▫N |  |
| Field Performance Subtotal (A) |  |  |  |  |  |

Scoring key:  Contact (Rocket touched + CW spin) = +20  |   Approach (Rocket not touched + CW spin) = +15  |   Correct Avoid = +15  |   Wrong-direction Avoid = +5  |   False contact = –10

Approach required: Robot must visibly drive toward object before any spin. Random spinning ≠ scored engagement.

| Performance Bonus (D) |  |  |
| --- | --- | --- |
| Perfect Contact (Rocket approached and touched) | ▫ Yes | +15 / 0 |
| Clean Avoid (no avoid object touched) | ▫ Yes | +15 / 0 |
| Bonus Total (D) |  |  |

Final Score

| A — Field Performance Subtotal |  |
| --- | --- |
| B — Model-Size Multiplier | × |
| A × B | = |
| C — mAP Bonus | + |
| D — Performance Bonus | + |
| TOTAL = (A × B) + C + D | = |

| Referee: | Signature: |
| --- | --- |